LSSD Automation PC Preload Specification – with Office 2013 as of 1/21/2014

APN: G1680-64048 (CPU LSSD Automation PC Kit)

Specification of the LSSD Automation PC Workstation

The PC used in the product offering will be configured as follows:

  - HP Z230 PC Workstation

  - Intel® Xeon® E3-1225v3 (3.20 GHz, 8 MB cache, 4 cores) CPU

  - Windows 7 (64-bit) SP1 preinstalled

  - 1 x 500 GB hard drive

  - 8 GB of RAM

  - Integrated Intel 1217LM PCIe GbE Controller; 2nd HP Serial Port Adapter; Intel Ethernet I210-T1 PCIe NIC card

The LSSD Automation PC shall be prepared using the Windows 7 (64-bit) SP1 Recovery DVD-ROM            ,  plus an “Autoloader” script which will configure some crucial settings in Windows 7 which allows proper functionality, including

- Removal of D: partition

  - Updating power options

  - Configuration of two LAN Cards

    - Local Area Connection network with DHCP

    - Sub Network with fixed IP of 192.168.0.1 on TCP/IPv4, updated power settings, disable TCP/IPv6

  - Creation of the default account “admin” with password “3000hanover” with administrator rights

  - PC will be delivered with System Preparation activated so that customer can accept the Microsoft End User License Agreement (EULA) and perform basic setup tasks (OOBE).

  - Change Windows Update Setting

In addition to these settings, the PC will be delivered with the following software pre-installed:

- Latest Agilent Supply Catalog

- Microsoft Office 2013 Files loaded onto C:\ForAgilentServiceProfessionalsOnly*

- Installation of ClamWin Free Antivirus**

- Update Adobe Reader

- MySQL

- DHCPSRV

Notes: *MS Office 2013 will not be completely installed nor activated.  Process will need to be completed by Agilent.  **An additional installation step will need to be completed by Agilent in order to move infected files to quarantine folder

The following labels will be affixed to the PC in order to help identify important information/items:

  - Voltage Warning Label

  - License Acceptance Label

  - House Label (Affixed to onboard NIC connection)