LSSD Automation Laptop Preload Specification – with Office 2013 as of 5/2/2014

APN: G5880-64009 (CPU LSSD Automation Laptop Kit)

Specification of the LSSD Automation Laptop Workstation

The laptop used in the product offering will be configured as follows:

  - HP ZBook 15 Mobile Workstation

  - Intel® Core™ i7-4600M (2.9 GHz, 4 MB cache, 2 cores)

  - 15.6" diagonal LED-backlit FHD UWVA IPS eDP anti-glare + Panel Self Refresh (PSR) (1920 x 1080)

  - Windows 7 (64-bit) SP1 preinstalled

  - 320 GB hard drive

  - 8 GB of RAM

- Intel Dual Band Wireless-AC 7260 802.11 ac (2x2)

- Bluetooth 4.0 Wireless Technology

- Linksys USB Ethernet Adapter USB300M (included in box)

The LSSD Automation laptop shall be prepared using the Windows 7 (64-bit) SP1 Recovery DVD-ROM            ,  plus an “Autoloader” script which will configure some crucial settings in Windows 7 which allows proper functionality, including

- Removal of D: partition

  - Updating power options

  - Configuration of two LANs

    - Local Area Connection network with DHCP

    - Sub Network with fixed IP of 192.168.0.1 on TCP/IPv4, updated power settings, disable TCP/IPv6 (the USB Ethernet adapter must be attached to any USB port before powering on the laptop in order to complete the setup)

  - Creation of the default account “admin” with password “3000hanover” with administrator rights

  - Laptop will be delivered with System Preparation activated so that customer can accept the Microsoft End User License Agreement (EULA) and perform basic setup tasks (OOBE).

  - Change Windows Update Setting

In addition to these settings, the laptop will be delivered with the following items pre-installed:

- Latest Agilent Supply Catalog

- Microsoft Office 2013 Files loaded onto C:\ForAgilentServiceProfessionalsOnly*

- Installation of ClamWin Free Antivirus**

- Installation of Linksys USB Ethernet Adapter driver

- Installation of External USB “CDM20814_Setup.exe” driver

- Update Adobe Reader

- MySQL

- DHCPSRV

Notes: *MS Office 2013 will not be completely installed nor activated.  Process will need to be completed by Agilent.  **An additional installation step will need to be completed by Agilent in order to move infected files to quarantine folder

The following labels will be affixed to the PC in order to help identify important information/items:

  - Agilent bundle PC sticker